# Dynamic Footlamp – Motion & Light Based Auto Control (No Microcontroller)

### Introduction

This project is a simple automatic footlamp that works without any microcontroller.
The lamp turns ON only when needed and reduces power waste.
It uses a motion sensor and an LDR to understand the surroundings and control the light automatically.

### Objective

The aim of this project was to build a low-cost footlamp that:

- Works only in the dark

- Detects movement

- Adjusts brightness according to the condition

- Gives three different modes (ON / AUTO / OFF)

- Works using only basic analog components

### Features of the Footlamp

This footlamp has 3 working modes:

#### 1. ON Mode

- Light stays ON at full brightness all the time.

- Good for manual use when continuous light is needed.

#### 2. OFF Mode

- Light remains completely OFF.

- Saves power when lamp is not required at all.

#### 3. AUTO Mode (Smart Mode)

AUTO mode has three brightness levels depending on the environment:

- Dark + No Motion → Light Slightly ON (Dim Mode)
 The lamp glows dim when it's dark but no one is nearby.
 This helps the footpath stay visible.

- Dark + Motion Detected → Full Brightness
 When someone walks near the sensor, the brightness increases and fully glows.
 This gives clear visibility for walking.

- Light Present (Daytime) → Lamp Completely OFF
 LDR senses brightness and keeps the lamp OFF during the day.

This mode saves maximum energy and makes the lamp smart and automatic.

### Components Used

- RCWL-0516 Microwave Motion Sensor

- LDR (Light Dependent Resistor)

- COB LED Light (12V)

- BC547 NPN Transistor

- 10 kΩ Base Resistor

- 1N4007 Diode

- External Mode Switch (3-way ON / AUTO / OFF switch)

- 12V DC Supply

### Working Principle

The whole system works using LDR + Motion Sensor + Transistor.

- LDR decides whether it is dark or bright.

- RCWL sensor checks if any person or movement is near.

- Transistor combines both signals and controls the COB LED brightness.

- The external switch lets the user choose ON, AUTO, or OFF mode.

There is no digital circuit or microcontroller involved — everything runs using simple analog behavior.

### Circuit Explanation

- The LDR and a resistor form a voltage divider, giving a signal based on light level.

- RCWL gives a HIGH output when motion is detected.

- Both signals go to the base of the BC547 transistor.

- The transistor adjusts brightness by controlling current through the COB LED.

- In AUTO mode, the transistor only activates the light based on sensor conditions.

- A diode protects the LED from reverse voltage.

### Values From Your Notes (Converted Cleanly)

- LDR dark: ~8kΩ

- LDR light: ~3.2kΩ

- Base resistor: 10 kΩ

- Base current (motion + dark): ~0.045 mA

- COB LED internal resistance: ~1.8 MΩ

These values ensure the transistor gets enough base current and switches smoothly between dim and bright states.

### Advantages

- Fully automatic

- No coding needed

- Low power consumption

- Easy to build

- Works well indoors and outdoors

### Applications

- Garden pathway

- Staircase lighting

- Parking area lights

- Hostel corridor lamps

- Room corner night lamp

### Result

The footlamp worked as expected.
In bright light, the lamp stayed OFF.
During night, the lamp glowed dim and became fully bright when movement was detected.
The three modes also worked properly without any delay or flicker.

### Conclusion

This project shows that even without microcontrollers, we can build a simple and smart automatic lighting system.
The combination of LDR, motion sensor, and transistor provides smooth control of brightness based on real conditions.
It is reliable, cheap, and useful for many everyday lighting needs.